# 5G NR OVERVIEW

Author: Vũ Công Hoàn

- 5G NR System Overview

- RAN (radio access network): responsible for all radio-related functionality. Like: scheduling, radio-resource handling, retransmission protocols, coding, various mutil-antena schemes. (provides the connection between the end user and the network)

- CN (core network):  responsible for function not related to the radio access. Like: authentication, charging functionality, setup end to end connection. ( is the backbone of the telecommunications network. Provides the backbone for the entire network, enabling connectivity to other network).

  - The 5G Core Network

- Service-based architecture:

  - Service and functionality provide by core network.

  - 5g network functionality to become more granular and decoupled. It becomes independently with other function.

- Support for network slicing:

  - Create multiple logical networks on a common hardware.

  - Each network slice can be configured to meet the requirements of various applications

- Control plane/user plane split:

  - They are independently about the capacity of the two.

  - User-plane: the process of sending and receiving user data, which are the main signal for communication.

  - User plane is gateway between the RAN and external internet.

  - User plane responsibilities include: packet routing and forwarding, packet inspection, QoS handling, packet filtering and traffic measurements.

  - Control-plane: a series of control process that are exchanged to establish communication and authentication.

  - Control plane include:

    - Session management function (SMF): IP address allocation for the device (user equipment (UE)), control of policy enforcement and general session management.

    - Access and mobility management function (AMF): control signal between core network and device, security user data, idle-state mobility and authentication.

  - Policy control function (PCF): responsible for policy rules

  - Unified data management (UDM) responsible for authentication credentials and access authorization.

  - Network exposure function (NEF)

  - NR repository function (NRF)

  - Authentication server function (AUSF)

  - Network slice selection function (NSSF)

  - Unified  data repository (UDR)

  - Application function (AF)

  - The Radio-Access Network (RAN)

Radio-access network (RAN): can have two types of nodes connected to the 5G core network.

- gNB, serving NR devices

- ng-eNB, serving LTE devices

- A radio-access network consisting of both ng-eNB and gNB is known as an NG-RAN.

- gNB or eNG is responsible for all radio-related function in one or more cells.

- gNB is a logical node and not a physical implementation.

- gNB connected to 5G core network by NG interface

- One gNB can be connected to multiple UPF/AMF for the purpose of load sharing and redundancy.

- gNB connected to other gNB by Xn interface.

- gNB can have two part :

  - A central unit (CU): support RRC, PDCP, SDPA protocols

  - One or more distributed units (DU): support RLC, MAC, PHY protocols.

- CU connected to DU by F1 interface

- DU connected to UE by Uu interface

- 5G NR Protocols Overview

## 2.1. Radio Protocol Architecture

- Non-access-stratum (NAS) used to manage the establishment of communication sessions and for maintaining continuous with UE as it moves.

- UE connected to AMF (core network) by NAS protocol.

- UE connected to gNB by RRC, SDAP, PDCP, RLC, MAC, PHY protocols.

## 2.2. User-Plane Protocols

- Service Data Application Protocol (SDAP)

  - Is responsible for QoS handling. In particular, SDAP will map a specific QoS Flow within a PDU Session to a corresponding Data Radio Bearer.

  - SDAP will mark the transmitted packets, ensuring that the packet receives the correct

- Packet Data Convergence Protocol (PDCP)

  - performs IP header compression, ciphering, and integrity protection.

  - handles retransmissions, in-sequence delivery, and duplicate removal in the case of handover.

  - For dual connectivity with split bearers, PDCP can provide routing and duplication.

  - There is one PDCP entity per radio bearer configured for a device.

- Radio-Link Control (RLC)

- is responsible for segmentation and retransmission handling.

- The RLC provides services to the PDCP in the form of RLC channels

- Medium-Access Control (MAC)

  - handles multiplexing of logical channels, hybrid-ARQ retransmissions

  - scheduling and scheduling-related functions

- Physical Layer (PHY)

  - handles coding/decoding, modulation/demodulation, multi-antenna mapping, and other typical physical-layer functions

### 2.1.1. Service Data Adaptation Protocol (SDAP)

- mapping between QoS and data radio bearer

- marking Qos identifier

- gNB connect EPC (NSA), SDAP is not used.

### 2.2.2. Packet-data convergence protocol (PDCP)

- performs IP header compression to reduce the number of bits to transmit.

- header-compression is based on ROHC.

- ciphering to protect against eavesdropping

- at the receiver, corresponding deciphering and decompression operations.

- handles retransmissions, in-sequence delivery, and duplicate removal in the case of handover.

- PDCP is in charge of distributing the data between the MCG and the SCG

### 2.2.3. Radio-Link Control (RLC)

- segmentation of RLC SDUs from the PDCP into suitably sized RLC PDUs

- handles retransmissions of incorrect received PDUs.

- Removal of duplicate PDUs.

- 3 modes:

  - Transparent mode:

    - RLC Header Added : No

    - Buffering: Buffering done only at Transmission

    - Functionality: No Segmentation and No Reassembly

    - Feedback Mechanism (ARQ): No feedback (No ACK/NACK for RLC PDU)

  - Unacknowledged mode:

    - RLC Header Added : Yes

    - Buffering: Buffering done both at Transmission and Reception

    - Functionality: Segmentation done at TX and Reassembly at RX side

    - Feedback Mechanism (ARQ): No feedback (No ACK/NACK for RLC PDU)

  - Acknowledged mode:

    - RLC Header Added : Yes

    - Buffering: Buffering done both at Transmission and Reception

    - Functionality: Segmentation done at TX and Reassembly at RX side

    - Feedback Mechanism (ARQ): Feedback mechanism is there (ACK/NACK for RLC PDU)

### 2.2.4. Medium-Access Control (MAC)

2.2.4.1. Logical Channels and Transport Channels

Logical-channel includes:

The MAC provides services to the RLC in the form of logical channels

- Control channel: used for transmission of control and configuration information necessary for operating an NR system

- Traffic channel: used for the user data

- Broadcast Control Channel (BCCH): transmission of system info from the network to all devices in cell.

- Paging Control Channel (PCCH): paging of devices whose location on a cell level is not known to the network. The paging message need to be transmitted in multiple cells

- Common Control Channel (CCCH): transmission of control information in conjunction with random access

- Dedicated Control Channel (DCCH): transmission of control information to/from a device. Use for configuration of a device.

- Dedicated Traffic Channel (DTCH): transmission of user data to/ from a device

Transport-channel includes:

From the physical layer, the MAC layer uses services in the form of transport channels.

Data on a transport channel are organized into transport blocks.

In each Transmission Time Interval (TTI), at most one transport block of dynamic size is transmitted over the radio interface to/from a device.

- Broadcast Channel (BCH): has a fixed transport format, provided by the specifications. It is used for transmission of parts of the BCCH system information

- Paging Channel (PCH): is used for transmission of paging information from the PCCH logical channel

- Downlink Shared Channel (DL-SCH):

  - is the main transport channel used for transmission of downlink data

  - It supports key NR features such as dynamic rate adaptation and channel-dependent scheduling in the time and frequency domains, hybrid ARQ with soft combining, and spatial multiplexing.

  - transmission of the parts of the BCCH system information not mapped to the BCH

- Uplink Shared Channel (UL-SCH): the uplink transport channel used for transmission of uplink data

- Random-Access Channel (RACH): it does not carry transport blocks

Part of the MAC functionality is multiplexing of different logical channels and mapping of the logical channels to the appropriate transport channels

2.2.4.2. Scheduling

- controls the assignment of uplink and downlink resources

- dynamic scheduling

- the gNB takes a scheduling decision. Once per slot, and sends scheduling information to the selected set of devices.

- the actual data transmission isn’t restricted to start or end at the slot boundaries

- Uplink and downlink scheduling are separated

- The downlink scheduler:

  - controlling which device(s) to transmit to

  - the set of resource blocks for DL-SCH.

  - Transport-format selection (selection of transport-block size, modulation scheme, and antenna mapping)

  - logical-channel multiplexing for downlink transmissions are controlled by the gNB

  - channel-state information (CSI) support downlink. It report channel quality.

- The uplink scheduler:

  - control which devices are to transmit on their respective UL-SCH.

  - uplink scheduling decision does not explicitly schedule a certain logical channel

  - the device is responsible for selecting from radio bearer(s)

2.2.4.3. Hybrid ARQ with Soft Combining

- provides robustness against transmission errors.

- is part of the MAC layer

- Hybrid ARQ is not applicable for all types of traffic (broadcast transmission).

- hybrid ARQ is only supported for the DL-SCH and the UL-SCH

- when receive of a transport block  decode and send ack bit to the transmitter.

- An asynchronous hybrid-ARQ protocol is used for both downlink and uplink

- Up to 16 hybrid-ARQ processes are supported.

### 2.2.5. Physical Layer (PHY)

- The physical layer is responsible for coding, physical-layer hybrid-ARQ processing, modulation, multi-antenna processing, and mapping of the signal to the appropriate physical time-frequency resources

- mapping of transport channels to physical channels

- provides services to the MAC layer in the form of transport channels

- A physical channel corresponds to the set of timefrequency resources

- each transport channel is mapped to a corresponding physical channel

- Physical Downlink Shared Channel (PDSCH): the main physical channel used for unicast data transmission.

- Physical Broadcast Channel (PBCH): carries part of the system information, required by the device to access the network.

- Physical Downlink Control Channel (PDCCH): is used for downlink control information, mainly scheduling decisions, required for reception of PDSCH, and for scheduling grants enabling transmission on the PUSCH

- Physical Uplink Shared Channel (PUSCH): There is at most one PUSCH per uplink component carrier per device.

- Physical Uplink Control Channel (PUCCH): send hybrid-ARQ, support downlink channel-dependent scheduling, and requesting resources to transmit uplink data.

- Physical Random-Access Channel (PRACH): is used for random access.

## 2.3. Control-Plane Protocols

- responsible for connection setup, mobility, and security.

- NAS connect AMF and UE. It includes authentication, security, and different idlemode procedures. responsible for assigning an IP address to a device.

- Radio Resource Control (RRC): responsible for handling the RAN related control-plane procedures. Includes:

  - Broadcast of system information necessary

  - Transmission of paging messages originating from the MME to notify the device about incoming connection requests.

  - Connection management, including setting up bearers and mobility

  - Mobility functions such as cell (re)selection.

  - Measurement configuration and reporting.

  - Handling of device capabilities

### 2.3.1. RRC State Machine

- three RRC states, RRC_IDLE, RRC_ACTIVE, and RRC_INACTIVE

- RRC_IDLE: No data transfer may take place as the device sleeps most of the time to reduce battery consumption

  - Broadcast of System Information

  - Cell re-selection mobility

  - DRX for CN paging configured by NAS

- RRC_CONNECTED: for communication between the device and the radio-access network are known to both entities

  - The connected state is intended for data transfer to/from the device, but can be configured (DRX)

  - Mobility is managed by the radio-access network.

  - 5GC - NG-RAN connection is established for UE

  - The UE AS context is stored in NG-RAN and UE

  - NG-RAN knows the cell which the UE belongs to

  - Transfer of unicast data to/from the UE

  - Network controlled mobility including measurements

- RRC_INACTIVE: can be seen as a mix of the idle and connected states.

  - Broadcast of system information

  - Cell re-selection mobility

  - Paging is initiated by NG-RAN

  - RAN based notification area is managed by NG-RAN

  - DRX for RAN paging configured by NG-RAN

  - The UE AS context is stored in NG-RAN and the UE

  - NG-RAN knows the RNA which the UE belongs to

- For the idle and inactive states, mobility is handled by the device through cell reselection,

- while for the connected mode, mobility is handled by the radio-access network based on measurements

### 2.3.2. Idle-State and Inactive-State Mobility

2.3.2.1. Tracking the Device

- devices are only tracked on a cell-group level

  - The network only receives new information about the device location if the device moves into a cell outside of the current cell group

  - broadcast over all cells within the cell group.

  - RAN Area Identifier (RAI)

  - Tracking Area Identifier (TAI).

  - When a device enters a cell( be not assigned by UE)update NAS.

2.3.2.2. Paging Message Transmission

- paging messages are provided by means of ordinary scheduled PDSCH transmissions

- a device is only supposed to wake up to monitor for paging messages.

- Paging messages are indicated by a specific PI-RNTI carried within the DCI

- Then the device demodulates and decodes the corresponding PDSCH to extract the paging message(s)

### 2.3.3. Connected-State Mobility

- to ensure that this connectivity is retained without any interruption or noticeable degradation as the device moves within the network.

- the device continuously searches for new cells both on the current carrier frequency and on different carrier frequencies

- the device does not make any decisions of its own when it comes to handover to a different cell

- the device reports the result of the measurements to the network